DT-ENG-DX471/TD7-V1 / 201904

2 WIRE HOME Wi-Fi SYSTEM

DX471-TD7

7" COLOR TFT Wi-Fi MONITOR

·  Please read this manual carefully before using the product you purchase,and keep it well for future use.

·  Please note that images and sketch maps in this manual may be different from the actual product.

Contents

1.Parts and Functions .................................................. 1

2.Mounting ................................................................... 2

3.Main Menu ................................................................ 3

4. Answering a Call ...................................................... 5

5.Panview & Zoom ....................................................... 6

6.Monitoring ................................................................. 6

7.Call Another Rooom or Apartment(Intercom) ............ 8

8.View Call Record....................................................... 10

9.Open Staircase Light................................................. 11

10.No Disturb Function ................................................ 12

11.Divert Call ................................................................ 13

12.Setting ..................................................................... 14

Enter setting interface .......................................................14

Ring tone setting ...............................................................15

General setting..................................................................15

Installer setting ..................................................................19

Wireless setting.................................................................26

About ...............................................................................31

13.Download Setup 2Easy APP................................... 32

14.Surveillance door station via 2Easy APP ................ 34

15.How It Works ........................................................... 34

16.Specification............................................................ 36

17.Precautions ............................................................. 36

1. Parts and Functions

LCD Screen

Speaker

Talk/Mon Button

POWGNDRINGSW-SW+

Power/No Disturb Indicator

Microphone

Unlock Button

ON

DIP

123456

Connection port

L1

L2

Key functions

-1-

| Speaker | Send out vioce from the visitor. |
| --- | --- |
| Talk/Mon button | Press to communicate hands free with visitor;Press to view the outdoor condition in standby mode. |
| LCD screen | Display the visitors' image. |
| Power/No Disturb indicator | Show the power status of the system.Show the no disturb status. |
| Microphone | Transmit audio from one station to other stations. |
| Unlock button | Press to release the door. |
| Connection port | Bus terminal, Setting the monitor's address. |

Terminal description

SW+,SW-: Extra door bell call but-

ton connection port.

POW GND RING SW- SW+

Ring,GND: Extra buzzer connec-

tion port.

ON            DIP

123456

SD

POW,GND: 12~16V extra power

connection port.

DIP switches: Total 6 bits can be

configured.

Bit1~Bit5: User Code setting

-37·

switch.

Bit6: Video impedance matching

-37·

switch.

L1

L2

USB-Wi-Fi adaptor

L1,L2: Bus line terminal.

2. Mounting

Mounting box

The unit

Mounting screw

x 2(included)

2 wire cable

Mounting bracket

The installation height is suggested to 145~160cm.

1. Use screws to fasten the mounting bracket to the wall with mounting box.

2. Connect the 2 wire cables to the unit.

3. Mount the unit to the mounting bracket, make sure the unit is absolutely

attached to the mounting bracket.

-2-

145~160 cm

3. Main Menu

The Main menu is your starting point for using all the applications on your moni-

tor.

Touch Unlock button, or touch anywhere of the screen on monitor in standby

mode, the Main menu will appear as follow:

Monitor

Touch it to view outdoor condition.

Touch this icon to directly

enable the indicated function.

Intercom

Touch it to enter intercom

selections.

Call Record

Touch it to see calling in/calling

out records or missed calls .

Current date and time.

Touch it to enter the date

time set interface.

Status bar

Light

Touch it to light up the

staircase light via RLC.

SIP config

Touch it to enter SIP

config interface.

Wi-Fi status

Touch it to enter

Wireless setting

options.

Setting

Press it to enter setup interface.

Close

Touch it to turn off the screen

and make the monitor in standby

mode.

Call Scene

Touch it to activate the

functions of No disturb, Divert

call.

Function status

| -3- Icon | Meaning | Description |
| --- | --- | --- |
|  | Missed call | Display when there is missed call unread.Touch it to review the missed call in shortcut. |
|  | No disturb | Display when activate the functionTouch it to enter Call Scene interface in shortcut. |
|  | Call transfer | Display when activate the functionTouch it to enter Call Scene interface in shortcut. |
|  | SIP server connection active and configured | Touch it to enter SIP server information interfacein shortcut. |
|  | SIP server connection disabled | Touch it to enter SIP server information interfacein shortcut. |
|  | SIP server connection active andnot connected | Touch it to enter SIP server information interfacein shortcut. |

While talking with a door station

Displays which door station

is calling.

Shows status of talking, unlocking

ect.

Red while recording.

Displays talking time.

00:10

Door Station1

5 direction pad button,

zoom&pantilt the picture.

Note: The    5 direction pad operation should make effective with fish-eye door station.

-4-

| Icon | Meaning | Description |
| --- | --- | --- |
|  | Wi-Fi connection active and notconnected | Touch it to enter Wi-Fi setting interface in shortcut. |
|  | Wi-Fi connection disabled | Touch it to enter Wi-Fi setting interface in shortcut. |
|  | Wi-Fi connection active and configured | Touch it to enter Wi-Fi setting interface in shortcut. |

4. Answering a Call

1. Touch it to answer the call.

To end the call touch it again.

2. If necessary, during the call

touch it to display the audio/

video adjustment icons.

3. Touch it to record image/

video.

4. Touch the icons to adjust

00:10

Door Station1

the parameters you want.

00:10

Door Station1

5

3

6

Adjust the image colour.

Adjust the image brightness.

Adjust the volume.

5. Touch it to open the door

lock for visitor.

00:10

Door Station1

The door lock key display

on screen briefly, to indicate

that the door lock has been

released.

-5-

5. Panview & Zoom

If wants to see greater detail about visitors,move to the desired position by

touching

on the screen to view the image in pantilt & zoom mode.

1. Touch it to activate the panview

and zoom function.

00:10

Door Station1

6. Monitoring

This section can be used to monitor your home by enabling one of the cameras

inside the apartment, the common areas (e.g. garages, garden), and at the

entrance.

1. Touch it to display the cameras

in your system.

-6-

2. Touch it to activate the desired

camera on Monitor interface            .

3. It is possible to “>” (display in

sequence) the other cameras.

4. Touch it to return the previous

menu.

5. Touch it to activate the specified

camera in shortcut.

The display shows the images recorded by the active camera.

6. If the camera belongs to an

Door Station, touch it to activate

00:10

Door Station 2

audio communication.

Touch it again to return the

Monitor interface            .

-7-

7. Call Another Room or Apartment (Intercom)

It is possible to communicate with another device installed in another room of the

apartment or with a device of another apartment.

1. Touch it to display the available

intercoms.

2. Touch it to display the available

namelist calls.

3. Touch it to activate the

communication with the desired

intercom.

Intercom

Namelist

Intercom

Namelist

Inner Call

Guard Station

Input Numbers

Namelist

HEBE 1

HEBE 2

HEBE 3

HEBE 4

HEBE 5

the call is active.

-8-

00:15

HEBE 1

4. Touch it to activate the

communication with another room.

Intercom

Intercom

Namelist

Inner Call

Guard Station

Input Numbers

5. Touch it to activate the

communication with guard station.

Intercom

Intercom

Namelist

Inner Call

Guard Station

Input Numbers

6. If the system does not have the

room number you want, you can enter

Intercom

other room number. Touch it to enter

input numbers interface.

Intercom

Namelist

Inner Call

Guard Station

Input Numbers

-9-

7. Input the room numbers you want.

123_

Note: when a call is received from the Door Station while the internal unit is in “intercom”

mode, the system follows the priority rules and ends the active call.

8. View Call Record

When there are one or more unread call records in the answering machine,

clicking the icon shows them.

1. Touch it to enter call record interface

and display the records. These records

include missed calls, incoming calls,

outgoing calls and playback.

2. Touch it to show the missed calls.

Missed call list:

-10-

Calling date&time

Calling source

Missed

Incoming

Outgoing

Playback

[2017/09/16 09:50] Door Station 1

[2017/09/16 11:25] Door Station 1

[2017/09/17 13:50] Door Station 2

[2017/09/17 18:37] Door Station 1

[2017/09/18 08:52] Door Station 2

The infomations you select is played back automatically.

3. Touch it to scroll interface.

1/223

4. Touch it to delete the image.

[2017/09/16 09:50] Door Station 1

9. Opening Staircase Light

Note: The staircase light should be connected on the system, and this function

should be supported by RLC. Please refer to RLC user instruction in detail.

1. Touch it to open the

staircase light.

-11-

10. No Disturb Function

If you don’t want to be disturbed, for example, at night. Activating the mute

function is necessary for you.

1. Touch it to enter call scene interface.

2. Touch it to set the no disturb mode: No

Disturb 8H and No Disturb Always.

CallScene

CallScene

Normal Use

No Disturb 8H

No Disturb Always

Divert Call If No Answer

Divert Call Always

No Disturb 8H: The calling from door station or other monitors will be forbidden for 8

hours.

No Disturb Alaways: The calling from door station or other monitors will be forbidden all

the time until the function is released.

Note:

* The no disturb indicator turns from green to red, to indicate that the no disturb function

has been activated.

* This function will perform immediately if selected,and the status bar on main menu

-12-

interface will show "

" icon.

11. Divert Call

The calling from door station can be diverted to your phone. please note that this

function should be supported by Wi-Fi or SIP network.

1. Touch it to enter call scene interface.

2. Touch it to set the divert call mode:

Divert Call If No Answer and Divert Call

CallScene

Always.

CallScene

Normal Use

No Disturb 8H

No Disturb Always

Divert Call If No Answer

Divert Call Always

Divert Call If No Answer: If nobody answers the call in 30s when receives call from

door station, diverts the call to the smart phone.

At this mode,the monitor will always open the screen and show image from door station

during diverted, if the monitor answers the call at this time,diverted will conclude at once.

Divert Call Always: When receives call from door station, diverts the call to smart phone

directly.

At this mode, the monitor will shut off the screen after diverting successfully, but you can

still use it in normal.

Note:

* If divert the call to smart phone via Sip sever, need to choose one.

* This function will perform immediately if selected,and the status bar on main menu

interface will show "

" icon.

-13-

12. Setting

Enter setting interface

1. Touch it to enter setting interface.

Call tune

Call Tune

General

Installer

Wireless

About

Ring volume

Ring time

Door1 calling tune

Door2 calling tune

Door3 calling tune

[04]

[35s]

[01]

[01]

[01]

2. Touch the icon for the desired customisation, choosing among:

-14-

Call Tune

General

Installer

Wireless

About

Ring volume - Ring time - Door1/2/3/4 calling tune - Doorbell calling tune -Intercom calling

tune -InnerCall calling tune -GuardCall calling tune

Date and time -Language -Monitor time -Shortcut setting -SD card - Screen on in divert -

Restore to default

Current address -Intercom enable - IPC Setting - Monitor lisht manage - System setting - FW

upgrade - Unlock time - Unlock mode - Auto reboot - Reboot

Wi-Fi switch -Wlan setting -Wireless device regist - SIP config - SIP tools

All kinds of device information

Ring tone setting

The ring call tone can be set individually to distinguish different calling sources.

Total 22 tones can be selected.

Scroll to next interface

1. Touch it to enter call tune setting

interface.

2. Touch the setting item to select the

call tone.

Call Tune

General

Installer

Wireless

About

Call Tune

Ring volume

Ring time

Door1 calling tune

Door2 calling tune

Door3 calling tune

[04]

[35s]

[01]

[01]

[01]

Ring volume: adjust the ringtone volume.

Ring time: adjust the ringtone time.

Door1/2/3/4 calling tune: call tone from door1/2/3/4 entrance panel.

DoorBell calling tune: call tone from doorbell.

Intercom calling tune: call tone from an intercom of another apartment.

InnerCall calling tune: call tone from an intercom in your apartment.

GuardCall calling tune: call tone from the Gurad unit.

General setting

This section contains the general settings and device information.

1. Touch it to enter general setting

interface.

Call Tune

General

Date and time

General

Installer

Wireless

About

Language

Monitor time

Shortcut setting

SD card

[000]

[300]

-15-

Date and time setting

The current date and time can be modified at any time.

General

Date time set

Call Tune

General

Installer

Wireless

Date and time

Language

Monitor time

Shortcut setting

General

Date mode:

Time mode:

Date:

Time:

MM_DD_YYYY

HOUR_12

xxxx_xx_xx

xx:xx

About

SD card

1. Touch it to enter the date time set interface.

2. Touch it to select the item you want to change.

Date mode: 2 display mode: month/date/year and date/month/year

Time mode: 2 display mode: HOUR_12 and HOUR_24.

Date: touch it to set the date .

Time: touch it to set the time.

Time zone: touch it to set the time zone in your country.

Time auto update: Disable: should set the time manually. Enable: can synchronize the

system time

Changing the display language

The default language can be customized. But you can change it to your desire

language at any time.

General

Language

Call Tune

General

Installer

Wireless

About

Date and time

Language

Monitor time

Shortcut setting

SD card

General

[000]English

[001]Franch

[002]German

[003]Spanish

1. Touch it to enter the language interface.

2. Select the target language, and then touch it to confirm.

*If the target language is not found in the window, touch > icon to scroll next interface.

-16-

Setting monitor time

The monitor time can be changed at any time. It’s available to set 30s, 60s, 90s,

120s, 150s, 180s, 210s, 240s, 270s, 300s.

* 30s is the default monitor time.

General

Monitor time select

Call Tune

General

Installer

Wireless

About

Date and time

Language

Monitor time

Shortcut setting

SD card

General

30(s)

60(s)

90(s)

120(s)

150(s)

1. Touch it to enter the monitor time select interface.

2. Select the monitor time you want, and then touch it to confirm.

Shortcut setting

In this section it is possible to select the 4 quick actions that will appear on the

home interface.

General

Shortcut set

Call Tune

General

Installer

Wireless

About

Date and time

Language

Monitor time

Shortcut setting

SD card

ShortcutSet1

ShortcutSet2

ShortcutSet3

ShortcutSet4

Remove shorcut

Monitor list

Namelist

Inner Call

Guard Station

1. Touch it to enter the shortcut set interface.

2. Select the shortcut set item you want.

3. Select the type of function for which you want to add a quick action.

4. Remove the quick action from the home interface.

-17-

SD card

If the device insert an SD card, the SD card info can be reviewed on the device.

General

SD Card

Call Tune

General

Installer

Wireless

About

Date and time

Language

Monitor time

Shortcut setting

SD card

General

SD information

Copy image

1. Touch it to enter the SD card interface.

2. Select the item you want, and then touch it to confirm.

SD information:SD card capacity.

Copy image: touch it to copy the recorded images to SD Card.

Screen on in divert

When the monitors set to "Divert call always" mode, in default mode, when

receive a calling, the monitors' can not display the image. But the settings can

be changed,you can set the monitors to panel on at the same time when divert a

call.

General

General

Call Tune

General

Installer

Wireless

About

Date and time

Language

Monitor time

Shortcut setting

SD card

Call Tune

General

Installer

Wireless

About

Screen on in divert

Restore to default

[Disable]

1. Touch it to scroll to next interface.

Screen on in divert

-18-

2. Touch it to enter the screen on in

divert interface.

3. Select the item you want, and then

touch it to confirm.

General

Disable

Enable

Restoring to default setting

The restore to default function allows the user to recover the settings to factory

setting.

General

General

Call Tune

General

Installer

Wireless

About

Date and time

Language

Monitor time

Shortcut setting

SD card

Call Tune

General

Installer

Wireless

About

Screen on in divert

Restore to default

1. Touch it to scroll to next interface.

Restore to default

2. Touch it to enter the restore to default

interface.

3. Select the item you want, and then

touch it to confirm.

General

Clear call record and pictures

Clear Wi-Fi settings

Restore general settings

Restore installer settings

Restore factory default

Notes: It has five items:Clear call record and pictures, Clear Wi-Fi settings,

Restore general settings, Restore installer settings, Restore factory default.

Installer setting

This section contains the device address setting and system settings.

1. Touch it to enter installer setting

interface.

Call Tune

General

Installer

Wireless

About

Installer

Current addtess

Intercom enable

IPC Setting

Monitor list manage

System settings

[01]

Enable

-19-

DIP Switcher Set Address

The DIP switches are used to set the user code for each monitor.Total 6 bits can

be configured.

·  Bit-1 to Bit-5 are used for user code setting. The value range is from 0 to 31,

which have 32 different codes for 32 apartments.

·  When multi monitors need to be installed in one apartment, these monitors

should use the same user code, and the master/slave mode should be set on

the monitor. (Details refer to the section of Setting Slave Monitor)

·  Bit-6 is bus line terminal switch, which should be set to “ON” if the monitor is at

the end of bus line, otherwise be set to “OFF”.

Bit-6 switch setting

ON 1    2    3    4    5    6

ON(1)

OFF(0)

ON

=

ON

=

This monitor is assigned with DIP setting instructions.Total 32 DIP codes.

Installer

-20-

Call Tune

General

Installer

Wireless

About

Current addtess

Intercom enable

IPC Setting

Monitor list manage

System settings

[01]

Enable

| Bit state | Setting | Bit state | Setting |
| --- | --- | --- | --- |
| Bit state | When monitoris not at theend of bus line. |  | When monitoris at the end ofbus line. |

| ON |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | 6 |

| ON |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | 6 |

Enabling/Disabling intercom function

It’s available to forbid or activate the intercom function.

Installer

Intercom enable

Call Tune

General

Current addtess

Intercom enable

[01]

Enable

Installer

Disable

Enable

Installer

Wireless

About

IPC Setting

Monitor list manage

System settings

*In default setting, intercom function is enable for the monitor.

IPC Setting

It’s available to set the related function of IP camera.

Installer

IPC Setting

Call Tune

General

Installer

Wireless

Current addtess

Intercom enable

IPC Setting

Monitor list manage

[01]

Enable

Installer

Add IPC by searching

Add IPC by manual

IPC list

IPC DHCP

[Enable]

About

System settings

Add IPC by searching: Add the IP camera by searching online.

IPC Setting

IPC searching

Installer

Add IPC by searching

Installer

.......

Add IPC by manual

IPC list

IPC DHCP

[Enable]

-21-

Add IPC by manual: Add the IP camera by manual.

IPC Setting

IPC login

Installer

Add IPC by searching

Add IPC by manual

IPC list

Installer

IPC IP:

User name: admin

User pwd:

admin

IPC DHCP

[Enable]

Channel:

2

Login

IPC list: IP camera list.

IPC Setting

IPC monitor

Installer

Add IPC by searching

Add IPC by manual

IPC edit

IPC delete

[192.168.xxx.xxx] HeroSpeed

IPC list

IPC DHCP

[Enable]

1. Touch it to edit the IP camera's

name.

IPC monitor

IPC monitor

IPC edit

[192.168.xxx.xxx] HeroSpeed

IPC edit

[192.168.xxx.xxx] HeroSpeed

?

IPC delete

IPC delete

2. Touch it to enter delete the IP camera status.

3. Touch it to delete the IP camera.

-22-

IPC DHCP:It’s available to forbid or activate the IPC DHCP.

IPC Setting

IPC DHCP

Installer

Add IPC by searching

Add IPC by manual

Playlist

Disable

Enable

IPC list

IPC DHCP

[Enable]

Monitor list manage

It’s available to manage the monitor list.

Installer

Mon select

Call Tune

General

Installer

Wireless

About

Current addtess

Intercom enable

IPC Setting

Monitor list manage

System settings

[01]

Enable

Mon select

[X]Door 1

[X]Door 2

[X]Door 3

[X]Door 4

[X]Cam 1

[X] means the function is available;

Mon select

[-] means the function is off and unavailable.

Mon select

[-]Door 1

[X]Door 2

[X]Door 3

[X]Door 4

[X]Cam 1

System setting

This section will be reserve.

-23-

Software upgrade

It’s available to upgrade the device software online/SD card.

Installer

Installer

Call Tune

Current addtess

[01]

Call Tune

FW upgrade

General

Installer

Wireless

About

Intercom enable

IPC Setting

Monitor list manage

System settings

Enable

General

Installer

Wireless

About

Unlock time

Unlock mode

Auto reboot

Reboot

[03]

[Open]

[Enable]

In the FW update interface, users can select

FW update

the Servers/SD card to upgrade.Then touch

"Check" to activate it.

Installer

Server

Download code

Server1[47.91.xx.xx]

847xxx

Check

Unlock time setting

It’s available to set the unlock time.

Installer

Installer

Call Tune

Current addtess

[01]

Call Tune

FW upgrade

General

Installer

Wireless

About

Intercom enable

IPC Setting

Monitor list manage

System settings

Enable

General

Installer

Wireless

About

Unlock time

Unlock mode

Auto reboot

Reboot

[03]

[Open]

[Enable]

-24-

The unlock time can be changed by

yourself at any time. it can be set from

01 to 99 seconds.

Unlock time

08

Unlock mode setting

It’s available to set the unlock mode.

Installer

Installer

Call Tune

Current addtess

[01]

Call Tune

FW upgrade

General

Installer

Wireless

About

Intercom enable

IPC Setting

Monitor list manage

System settings

Enable

General

Installer

Wireless

About

Unlock time

Unlock mode

Auto reboot

Reboot

[03]

[Open]

[Enable]

There are two unlock modes: 1.power-

Unlock mode

on- to-unlock type=Open

2.power-off-to-unlock=Close.

Installer

Open

Close

Reboot

It’s available to Auto reboot ,or Reboot the device.

Installer

Installer

Call Tune

Current addtess

[01]

Call Tune

FW upgrade

General

Installer

Wireless

About

Intercom enable

IPC Setting

Monitor list manage

System settings

Enable

General

Installer

Wireless

About

Unlock time

Unlock mode

Auto reboot

Reboot

[03]

[Open]

[Enable]

Installer

Call Tune

FW upgrade

General

Installer

Wireless

About

Unlock time

Unlock mode

Auto reboot

Reboot

[03]

[Open]

[Enable]

?

-25-

Wireless setting

SD card

WIFI adaptor

Wi-Fi monitor(DX471)

1. Touch it to enter wireless setting

3P-USB cable for

FW updating

Wireless

interface.

Call Tune

General

Wifi switch                                                [ON]

Wlan setting

2. Touch it to activate the function, make

sure the switch is ON.

Installer

Wireless

About

Wireless device regist

SIP config

SIP tools

[00]

3. Touch it to enter wlan setting interface.

4. Touch it to scroll the interface.

Lan setting

5. Touch it to select the domestic network

to which you want to connect the device.

Wireless

Input SSID and PWD

VTKAP03

zhongshi

MagicCyclingGuest

360

6. Touch it to enter the input password

interface.

Wireless

SSID: VTKAP03

PWD:

Auto connect

Connect

-26-

7. Enter the password (if required).

8. Touch it to activate the connection, and

return to Wi-Fi information interface.

9. Go to Connect/Disconnect item.

PWD:

vtk_

Wireless

SSID: VTKAP03

PWD: vtk@123

Connect/Disconnect

Make slave Wi-Fi monitor to be registered with master Wi-Fi

monitor

On master Wi-Fi monitor and slave Wi-Fi monitor, go into wireless setting

interface.

1. Touch it to enter the device manage

interface.

Call Tune

General

Wireless

Wifi switch                                                [ON]

Wlan setting

Installer

Wireless

About

Wireless device regist

SIP config

SIP tools

[00]

-27-

2. Touch it to enter the master / slave register interface.

master Wi-Fi monitor

Device manage

slave Wi-Fi monitor

Device manage

Wireless

Wireless

Register new slave

Master register

My ID: 040019b93fa9/201

Receive slave:

Slave ID:------------ / ---

Wireless

Wireless

Register new master

Slave register

My ID: 040019b83ea9/201

Registered to:

Master ID:------------ / ---

Unegistered

Waiting for Slave's application

SIP config

It’s available to divert the call to mobile while out of home via SIP server.

On the call scene interface

CallScene

1. Touch it to select the divert call mode:

Divert Call If No Answer or Divert Call

Always.

CallScene

Normal Use

No Disturb 8H

No Disturb Always

Divert Call If No Answer

Divert Call Always

-28-

On the wireless setting interface

2. Touch it to enter the SIP config.

Setting

Sip config

Call Tune

General

Wifi switch                                                [ON]

Wlan setting

Wireless

USER NAME

PASSWORD

0e0019axxx

230**

Installer

Wireless

About

Wireless device regist

SIP config

SIP tools

[00]

DOMAIN

Manual registration

Use Default

2easyip.com

Sip config

Sip config

Wireless

USER NAME

PASSWORD

DOMAIN

0e0019axxx

230**

2easyip.com

Wireless

USER NAME

PASSWORD

DOMAIN

0e0019axxx

230**

2easyip.com

Manual registration

Use Default

Manual registration

Use Default

Registered

3. Touch it to restore all settings on “SIP config” and create 2 SIP accounts, one

for DX monitor and one for 2Easy APP. Server will apply to restore two accounts’

password to default, and register it. Only when:

If your DX monitor is not brand new, we suggest you touch “Use default”

before testing.

2Easy APP is not able to register on server

4. Touch it, DX monitor will apply to clean all SIP “contact” on server, and also

will try to register itself to server, with this function it will helps when:

2Easy APP is not able to get push notification.

DX monitor has connect with Wi-Fi, but “Server” icon with “!” or “x” beside.

-29-

SIP tools

SIP tools could test divert function and with an unattended door station.

1. Touch it to enter the SIP tools page.

Call Tune

General

Wireless

Wifi switch                                                [ON]

Wlan setting

Installer

Wireless

About

Wireless device regist

SIP config

SIP tools

[00]

2. While if your test with pressing door

station doesn’t works well, you could also

try with auto-test.Touch it to enter call

testing page.

Wireless

SIP tools

SIP call testing

SIP call statistics

Clear SIP call statistics

View SIP call logs

Copy SIP call logs to SD

“SIP call testing count” from 1 to 99,

means test automatically 1-99 times;

Set “Call interval” from 1 to 10 min,

means system will wait 1-10 min between

2 calling;

Keep “Retry timing” as default, 12sec

Wireless

Call Testing

SIP call testing count

Call interval

Retry timing

Video quality

Start SIP call testing

[1]

[3min]

[12sec]

[0]

Set “Video quality” to 0, means lowest

required to network;

3. After all settings have done, touch “Start SIP call testing”, DX monitor will

immediately divert a calling to mobile.

-30-

In SIP tools page, you can also know the items "SIP call statistics", "Clear SIP

call statistics", "View SIP call logs", "Copy SIP call logs to SD", "View registered

Phones".

About

1. Touch it to enter about interface.

Setting

Call Tune

General

Installer

Wireless

Device Type:

S/W Ver:

Serial No:

Node ID:

DX471

V1.0.0.171110111114

040019b93333

200

About

IP Address:

192.168.01.100

linked

Device Type: the device name.

S/W Ver: the sofeware version.

Serial No: the device serial number.

Node Id: Node ID number.

IP Address: the device IP address.

H/W Address: the device Mac address.

Subnet Mask: the device subnet mask.

Default Route: the device default route.

DT Address: the device code number.

M/S Code: show the device is a master monitor or sub monitor.

Online: show the numbers of sub monitor.

-31-

13. Download Setup 2Easy APP

1. Scan and download 2Easy APP, or search “2Easy” on APP store.

IOS

Android

2. Permit “2Easy” with internet access and camera, on DX monitor, press “SIP

Server” icon on main menu, and keep in  “SIP config”, on 2Easy APP, press on

top-right, to scan QR code on DX monitor, and press “Save”.

-32-

3. Disable “IPV6” on 2Easy APP

Most of internet provide doesn’t support IPV6 feature, please manually disable it

with follow steps:

Press here

to disable

-33-

4. After login SIP account on 2Easy APP, check on status bar, or press on the

“Registered” area to manually register, until 2Easy APP display:

14. Surveillance door station via 2Easy APP

On 2Easy APP, press on “Monitor” and wait for a few second (Due to 2-Wire

communication will take around 12 second to get video), blue screen is normally

due to the DX monitor is verifying the password and monitor code.

15. How It Works

Two working modes

There are 2 working modes depend on whether your mobile phone is covered

with same Wi-Fi network or not, and DX master monitor can be regarded as a

gateway device.

-34-

Local mode--at home with same Wi-Fi network

· No need SIP sever and account log in

· Both Wi-Fi sub Monitor and Phone in Local mode work as wireless slave

· monitors

Home router

2-wire System

RFCARD

2

2

PC6A

Wi-Fi master

montior

Wi-Fi sub

monitor

Wireless terminal

(smart phone)

In case of calling, Wi-Fi sub Monitor and Phone will ring simultaneously, one device

answer the call will make others turn off automatically.

Divert mode--away from home with call divert

· SIP server and account are a must(A factory account is preset)

SIP SERVER

2-wire System

RFCARD

2

2

PC6A

Gateway

WIFI/4G

Wireless terminal

(smart phone)

· Call scenes(No disturb / If no answer / Divert always) in away home

Door Station Calling

No Disturb

No rings at all

If No Answer

Slaves Ringing

Mobile Ringing

Divert Immediately

Mobile Ringing

Note: Any one off hook, others will be

closed automatically.

-35-

16. Specification

● Power supply:

● Power consumption:

● Monitor screen:

● Display Resolutions:

DC 20~28V

Standby 1.8W; Working 8.16W

7 Inch digital color TFT

800*3(R, G, B) x 480 pixels

● Video signal:

● Wiring:

● Dimension:

1Vp-p, 75Ω, CCIR standard

2 wires, non-polarity

161(H)×174(W)×22(D)mm

17. Precautions

1) All components should be protected from violence vibration. And not allow to

be impacted, knocked and dropped.

2) Please do the cleanness with soft cotton cloth, please do not use the organic

impregnant or chemical clean agent. If necessary, please use a little pure water

or dilute soap water to clean the dust.

3) Image distortion may occur if the video monitor is mounted too close to

magnetic field e. g. Microwaves, TV, computer etc.

4) Please keep away the monitor from wet, high temperature, dust, caustic and

oxidation gas in order to avoid any unpredictable damage.

5) Must use the right adaptor which is supplied by the manufacturer or approved

by the manufacturer.

6) Pay attention to the high voltage inside the products, please refer service only

to a trained and qualified professional.

-36-

Note

-37-

DT-ENG-DX471/TD7-V1

The design and specifications can be changed without notice to the user. Right

to interpret and copyright of this manual are preserved.